16-CH Color Full Duplex Multiplexer

Instruction Manual

707-V1.5(S)

Index:

| 1. | Safety Warning |  |  | 3 |
| --- | --- | --- | --- | --- |
| 2. | Introduction |  |  | 3 |
| 3. | Features |  |  | 4 |
| 4. | Specification |  |  | 5 |
| 5. | Front Panel Keypad |  |  | 6 |
| 6. | Back Panel Connection |  |  | 10 |
| 7. | Menu Setting |  |  | 12 |
|  | 7.1 | Menu |  | 12 |
|  | 7.2 | Title Display |  | 12 |
|  | 7.3 | Alarm Duration |  | 13 |
|  | 7.4 | Dwell Time |  | 13 |
|  | 7.5 | VCR Record Time |  | 13 |
|  | 7.6 | Alarm VCR Record Time |  | 14 |
|  | 7.7 | INT Audible Alarm |  | 14 |
|  | 7.8 | EXT Audible Alarm |  | 14 |
|  | 7.9 | Event Message Latch |  | 14 |
|  | 7.10 | Security Lock |  | 15 |
|  | 7.11 | Set Time |  | 15 |
|  | 7.12 | Event List |  | 16 |
|  | 7.13 | Baud Rate |  | 16 |
|  | 7.14 | ID |  | 16 |
|  | 7.15 | Remote Interface |  | 16 |
|  | 7.16 | System Reset |  | 16 |
| 8. | Channel Setup |  |  | 17 |
| 9. | Motion Detection Setup |  |  | 19 |
| 10. | Monitor Operation |  |  | 21 |
| 11 | Auto Switching |  |  | 22 |
| 12. | VCR Playback Operation |  |  | 23 |
| 13. | External Alarm Input and Video Loss Signal |  |  | 24 |
| 14. | VCR Time Setting |  |  | 24 |
| 15. | Record Time Flash Rate Chart |  |  | 25 |
| 16. | Remote Control Function |  |  | 26 |
| 17. | Connection |  |  | 27 |
| 18. | Rack Mount Installation |  |  | 28 |

- Safety Warning

Please take a few minutes to review below safety precautions before operation.

  - Please lift and place this equipment gently.

  - Do not expose this equipment under straight sunlight environment.

  - Please keep this product away from the liquid or water.

  - When measuring current, turn the power off before connecting the meter in the

circuit.

  - Do not unplug the power connector before turning the power off.

  - Do not overload outlets and extension cords as this may cause fire or electric

shock.

  - Do not attempt to open the cover of this equipment, dangerous voltage of other
 hazards might occur while removing the cover.

  - Do not try to repair the product inner parts, any failure procedures might cause
 un-repairable damages (artificial damage will not be included in warranty promises).

  - Do not turn the Power On and Power Off during short time period (within 3 seconds)
 to let the product have good reliability.

2. Introduction

Thank you for choosing this high quality / Multi-function / color / full duplex Multiplexer. It can connect up to 16 cameras.  Before attempting to connect or operate this product, please read the safety warning and the instructions carefully. Any incorrect connection or operation might cause harm or damage to this product.

3. Features

  - Support NTSC and PAL

  - On screen menu display.

  - Full Duplex Multiplexing

- 16 channels camera can record/live display/playback at the same time.

- 16 channels loop back terminal.

  - Resolution

| Screen Modes | ###### Resolution |
| --- | --- |
| Full screen | 704(H) x 468(V) <NTSC> / 704(H) x 564(V) <PAL> |
| 4 channels | 352(H) x 234(V) <NTSC> / 352(H) x 282(V) <PAL> |
| 9 channels | 224(H) x 156(V) <NTSC> / 224(H) x 188(V) <PAL> |
| 16 channels | 176(H) x 117(V) <NTSC> / 176(H) x 141(V) <PAL> |

  - Independent main and call monitor outputs allow simultaneous multicamera and full screen viewing.

  - Multi Screen Display Mode

- Below display modes are selectable while surveillance & VCR play back:

  - Picture-In-Picture (PIP) is available in live and VCR playback modes,

Zoom in & out can be up to 2 X 2 in live and VCR playback modes.

  - 16 channels are swappable and independent title generator (up to 6 characters) for each channel.

  - Simultaneous digital video motion detection using 15x12 target (NTSC) 15x14 target (PAL) motion detection grids for each video channel to set different options.

  - Video Adjustable on Each Channel

  - Re-flash Rate

- Full screen : Real time display

- 16 channels: 0.133 sec/channel

  - Alarm Input Function & Output Function

- 16 channels alarm input, ALARM display and one alarm output.

- Video loss detected in each channel.

- Records 50 events.

  - Power-loss memory function: set up parameter will remain while power failure.

  - Remote function: it can be controlled by external host through RS-232Co or RS-485.

  - On Screen Display and RTC (Real time clock) Function.

  - Call Monitor

### 4. Specification

| Camera Input Signal | Composite video signal 1 Vp-p 75Ω BNC, 16 channels |
| --- | --- |
| # VCR Input Signal | Composite video signal 1 Vp-p 75Ω BNC S-Video Y 1 Vp-p 75Ω. C 0.8 Vp-p 75Ω S JACK |
| # Camera Loop Back | Composite video signal 1 Vp-p 75Ω BNC, 16 channels |
| Main Monitor Output | Composite video signal 1 Vp-p 75Ω BNC |
| Call Monitor Output | Composite video signal 1 Vp-p 75Ω BNC |
| # VCR Output | Composite video signal 1 Vp-p 75Ω BNC S-Video Y 1 Vp-p 75Ω. C 0.8 Vp-p 75Ω S JACK |
| # Motion Detect Area | 15 * 12 targets per camera (NTSC) 15 * 14 targets per camera (PAL) |
| # Motion Detect Sensitivity | 256 Levels |
| # Video Loss Detection | Yes |
| # Picture Freeze | Yes |
| # Refresh Rate | 30 Frames / Sec. |
| # Recording Rate | 15 Pictures/Sec.. |
| # Dynamic Recording Priority | Yes (Programmable) |
| # Dwell Time | Programmable (1~10 Sec) |
| # Picture in Picture | Yes (Movable) |
| # Key Lock | Yes |
| # Picture Zoom | 2*2 (Movable) |
| # Camera Title | 6 |
| # Video Adjustable on Channels | Color/Contrast/Brightness Adjustable |
| ## Alarm Input | TTL input, Hi (5V), Low (GND) |
| ## Alarm Output | COM, N.O. |
| ## Alarm History Log | 50 Events |
| ## Remote Control | RS232 or RS485 |
| VCR Trigger Output | Yes (TTL output) |
| Time Display Format | YY/MM/DD, DD/MM/YY, MM/DD/YY, OFF |
| Power Source | DC12V, 1.25A |
| Power Consumption | <10W |
| #### Operation Temperature | 0℃~ 50℃ |
| Operation Humidity | 30% ~ 90% |
| RS-232C / RS-485 (bps) | 115200、57600、19200、9600、4800、3600、2400、1200 |
| Dimensions (mm) | 432 (W) x 76(H) x 311(D) |
| Weight | 5,200g |

5. Front Panel Keypad

Some buttons on front panel have dual functions.  The primary function is listed first, followed by the secondary function (shown in parentheses).  Indicator lights (LEDs) are on each button.

5.1 MENU : Press ENTER+ ESC simultaneously to enter the menu mode

5.2 ENTER: Press ENTER to enter system setup of main item selection.

5.3 ESC: Press ESC to exit system setup of main item selection.

5.4 FREEZE:

5.4.1 Press FREEZE button to freeze all screen channels.

Press ESC to release.

5.4.2. Press FREEZE + ENTER simultaneously then press Camera Select (1-16) button to confirm the Freeze location. (The freeze location number will be displayed on monitor screen. The Camera Select (1-16) is to confirm the location, not camera channel).  is shown on the right of the location.  Press the same Camera Select (1-16) again to release freeze function.

  - LIVE

5.5.1  Press LIVE button to setup all of the screen channels on the screen as LIVE

display. Then press the below button for desirable screen modes display.

5.5.1.1  Press      :  to setup the 4 channels LIVE display screen.

5.5.1.2  Press      : to setup the 9 channels LIVE display screen.

5.5.1.3  Press      : to setup the 16 channels LIVE display screen.

5.5.1.4  Press       +4,7,9,10,13,16 : to setup the 4,7,9,10,13,16 channels
LIVE display screen.

5.5.1.5 Press Camera Select (1-16) directly to get the different camera full screen

5.5.2  Under LIVE display screen, press LIVE + ENTER simultaneously, and press     or      to select the appointed LIVE display screen, and press Camera

Select (1-16) to confirm the selected LIVE display camera.

Press ESC to exit.

5.5.3  The VCR video can be displayed with LIVE video simultaneously on multi-screen display mode.

5.5.3.1  Under LIVE display screen, press VCR + ENTER simultaneously, and press     or     to select the appointed VCR display screen, and press Camera

Select (1-16) to confirm the selected VCR display camera.

5.5.3.2  Press ESC to exit.

5.6 VCR

5.6.1 Press VCR button to setup all of the screen channels on the screen as VCR

display. Then press the below button for desirable screen modes display.

5.6.1.1 Press      : to setup the 4 channels VCR display screen.

5.6.1.2 Press      : to setup the 9 channels VCR display screen.

5.6.1.3 Press      : to setup the 16 channels VCR display screen.

5.6.1.4 Press       +4,7,9,10,13,16 : to setup the 4,7,9,10,13,16 channels
VCR display screen.

5.6.2  Under VCR display screen, press VCR + ENTER simultaneously, and press                or     to select the appointed VCR display screen, and press Camera Select

(1-16) to confirm the selected VCR display camera.

Press ESC to exit.

    - The LIVE video can be displayed with VCR video simultaneously on multi-screen

display.

      - Under VCR display screen, press LIVE + ENTER simultaneously button then press      or     to select the appointed LIVE display screen, and press Camera Select (1-16) to confirm the selected LIVE display camera.

5.6.3.2   Press ESC to exit.

5.7 ZOOM

Press ZOOM button to enlarge the main monitor picture display. It displays 2 x 2 zoom picture on main picture and a small window insert. The insert window contains a movable 1/4 sized view of the appointed camera.

5.7.1  Press the “▲, ▼,◄ and ►” arrow buttons to move the zoom pointer.       The multiplexer moves the 2 x 2 display accordingly.

5.7.2  Press Camera Select (1-16) to select the appointed camera LIVE display in ZOOM mode.

5.7.3  Press Camera Select (1-16) then press VCR + ENTER simultaneously to select the appointed camera VCR display in ZOOM mode.

5.7.4  Press ESC to exit.

  - Picture in Picture (PIP)

Press PIP       button to enter PIP display screen. The PIP format displays a full screen “background” picture with a 1/16 size screen ‘insert”.

5.8.1  Press       button to display the last selected PIP mode.

5.8.2  Press the “▲, ▼,◄ and ►” arrow buttons to move the insert screen.

5.8.3  The LIVE video can be displayed with VCR video simultaneously in PIP mode.

5.8.3.1  Press VCR + ENTER simultaneously then press the      or     button to select VCR display on full screen or insert screen, and press Camera Select (1-16) to select the appointed camera.

5.8.3.2  Press ESC to exit.

5.8.3.3  Press LIVE + ENTER simultaneously then press the      or     button to select LIVE display on full screen or insert screen, and press Camera Select (1-16) to select the appointed camera.

5.8.3.4 Press ESC to exit

5.9 ◄ Left arrow (2x2      )

5.9.1  4 channels display mode

5.9.2  To move leftward in on-screen menu after enter main item selection

5.9.3  To select system setup in on-screen menu after enter main item selection

5.10 ► Right arrow (3x3      )

5.10.1  9 channels display mode

5.10.2  To move rightward in on-screen menu after enter main item selection

5.10.3  To select system setup in on-screen menu after enter main item selection

5.11 ▲ Up arrow (4x4      )

5.11.1  16 channels display mode

5.11.2  To move upward in on-screen menu for main item selection

5.11.3  To select system setup after enter main item selection

5.12 ▼ Down arrow (      )

5.12.1  Multi-screen display select mode. After pressing ▼, the LED of selectable
 button will lighten. Continue to press the button for desirable screen modes
 display.  (If no selection will be made, press ESC again to exit selection)

      - Press “4”:  4 channels display mode

      - Press “7”:  7 channels display mode

      - Press “9”:  9 channels display model

      - Press “10”: 10 channels display mode

      - Press “13”: 13 channels display mode

      - Press “16”: 16 channels display mode

5.12.2  To move downward in on-screen menu for main item selection

5.12.3  To select system setup after enter main item selection

  - Camera Select (1-16)

    - Press the Camera Select (1-16) to select appointed camera to display on full
 screen mode.

### 6. Back Panel Connection

6.1  Loop

Connect video signal between Input port and Loop port to make a loop.

Connect up to 16 cameras

  - Camera Input: Connect up to 16 cameras.

  - VCR Out: Connect this output port to the VCR input port.

  - VCR In: Connect this input port to the VCR output port.

  - Monitor: Main monitor – live display/VCR playback display

  - Call: Call monitor out – auto switch channel

  - SVHS Out: Connect this output port to SVHS input of VCR

  - SVHS In: Connect this input port to SVHS output of VCR

  - Power Input: This is to connect DC12V, 1.25A power source

6.10  On/Off

On: power on

Off: power off

  - Alarm Connector

6.11.1  25 pin com port

6.11.2   9 pin com port

  - GND (Ground)

7. MENU SETTING

Press “ENTER+ ESC” simultaneously to enter the menu system setup, shown as Figure 1  (Menu will always display on Main monitor):

Figure 1 System Setup

| ► | TITLE DISPLAY | ON | OFF |  |
| --- | --- | --- | --- | --- |
|  | ALARM DURATION | 02 SECOND |  |  |
|  | DWELL TIME | 02 SEC |  |  |
|  | VCR RECORD TIME | 002 HOUR |  |  |
|  | ALARM VCR RECORD TIME | 002 HOUR |  |  |
|  | INT AUDIBLE ALARM | ON | OFF |  |
|  | EXT AUDIBLE ALARM | ON | OFF |  |
|  | EVENT MESSAGE LATCH | ON | OFF |  |
|  | SECURITY LOCK | ON | OFF |  |
|  | SET TIME | 00/01/01 | 00:00:00 | Y/M/D |
|  | EVENT LIST |  |  |  |
|  | BAUD RATE | 9600 BPS |  |  |
|  | ID | 000 |  |  |
|  | REMOTE INTERFACE | RS485 | RS232C |  |
|  | SYSTEM RESET |  |  |  |

  - ENTER + ESC (on-screen menu will auto jump out to normal screen after 60
seconds of no action)

    - Press “ENTER+ ESC” simultaneously to enter the menu system setup

7.1.2  Press “▲ or▼” to move arrow up and down for main item selection.

7.1.3  Press “ENTER” to confirm selection.

7.1.4  Press “ESC”  to confirm and return to main item selection

7.1.5  Press “ESC”  to exit on-screen menu.

  - TITLE DISPLAY

    - Press “ENTER+ ESC” simultaneously to enter the menu system setup

7.2.2  Press “▼” to move arrow down to TITLE DISPLAY

7.2.3  Press “ENTER” to enter system setup of TITLE DISPLAY

7.2.4  Press “◄ or ►” to set internal alarm ON or OFF

7.2.5  Press “ESC” to confirm and return to main item selection.

7.2.6  Press “ESC” to exit on-screen menu.

  - ALARM DURATION

It allows you to set the minimum multiplexer alarm time. The ALARM DURATION setting determines how many seconds or minutes the multiplexer remains in the alarm state after an alarm event occurs.

    - Press “ENTER+ ESC” simultaneously to enter the menu system setup

7.3.2  Press “▼” to move arrow down to ALARM DURATION

7.3.3  Press “ENTER” to enter system setup of ALARM DURATION.

7.3.4  Press      or     to set alarm duration time when alarm is activated.

7.3.4.1  Setting 1 ~ 59 seconds and 1 ~ 30 minutes

7.3.5  Press “ESC” to confirm and return to main item selection.

7.3.6  Press “ESC” to exit on-screen menu.

  - DWELL TIME

The DWELL TIME setting determines the rate at which the multiplexer sequences the cameras on the monitor.

    - Press “ENTER+ ESC” simultaneously to enter the menu system setup

7.4.2  Press “▼” to move arrow down to DWELL TIME.

7.4.3  Press “ENTER” to enter system setup of DWELL TIME.

7.4.4  Press the     or     button to setup the duration time for auto switch channel
in auto switch display screen: 1~10 seconds.

7.4.5  Press “ESC” to confirm and return to main item selection

7.4.6  Press “ESC” to exit on-screen menu.

7.5 VCR RECORD TIME: allows you to enter the VCR’s normal recording speeds.

7.5.1  Press “ENTER+ ESC” simultaneously to enter the menu system setup

7.5.2  Press “▼” to move arrow down to VCR RECORD TIME.

7.5.3  Press “ENTER” to confirm selecting VCR RECORD TIME.

7.5.4  Press     or      to select recording period

In NTSC system: selection of recording hours : 002H、006H、008H、012H、015H、016H、018H、020H、024H、036H、040H、048H、060H、072H、080H、084H、120H、160H、168H、240H、360H、480H、720H、960H

In PAL system: selection of recording hours : 003H、006H、012H、015H、018H、024H、030H、036H、048H、060H、072H、090H、120H、168H、240H、480H、720H、960H

    - Press “ESC” to confirm and return to main item selection

    - Press “ESC” to exit on-screen menu.

  - ALARM VCR RECORD TIME:

It allows you to enter the VCR’s alarm recording speeds.

    - Press “ENTER+ ESC” simultaneously to enter the menu system setup

    - Press “▼” to move arrow down to ALARM VCR RECORD TIME.

    - Press “ENTER” to confirm selecting ALARM VCR RECORD TIME.

    - Press      or      to select recording period

In NTSC system: selection of recording hours : 002H、006H、008H、012H、015H、016H、018H、020H、024H、036H、040H、048H、060H、072H、080H、084H、120H、160H、168H、240H、360H、480H、720H、960H

In PAL system: selection of recording hours : 003H、006H、012H、015H、018H、024H、030H、036H、048H、060H、072H、090H、120H、168H、240H、480H、720H、960H

    - Press “ESC” to confirm and return to main item selection

7.6.6 Press “ESC” to exit on-screen menu.

  - INT AUDIBLE ALARM: to set up internal alarm

    - Press “ENTER+ ESC” simultaneously to enter the menu system setup

    - Press “▼” to move arrow down to INT AUDIBLE ALARM.

    - Press “ENTER” to enter system setup of INT AUDIBLE ALARM.

    - Press “◄ or ►” to set internal alarm ON or OFF

      - Select ON: the buzzer will be activated

      - Select OFF: the buzzer will not be activated

    - Press “ESC” to confirm and return to main item selection.

7.7.6  Press “ESC” to exit on-screen menu.

  - EXT AUDIBLE ALARM: to set up external alarm

    - Press “ENTER+ ESC” simultaneously to enter the menu system setup

    - Press “▼” to move arrow down to EXT AUDIBLE ALARM.

    - Press “ENTER” to enter system setup of EXT AUDIBLE ALARM.

    - Press “◄ or ►” to set external alarm ON or OFF

    - Press “ESC” to confirm and return to main item selection.

7.8.6  Press “ESC” to exit on-screen menu.

  - EVENT MESSAGE LATCH

It controls the way the External Alarm    or Video Loss    graphic is displayed on monitor screen.

    - Press “ENTER+ ESC” simultaneously to enter the menu system setup

7.9.2  Press “▼” to move arrow down to EVENT MESSAGE LATCH.

7.9.3  Press “ENTER” to enter system setup of EVENT MESSAGE LATCH

7.9.4  Press “◄ or ►” to set external alarm ON or OFF

ON: The monitor screen displays the External Alarm     or Video Loss     graphic on the monitor and the LED of Camera Select (1-16) button will flash.

OFF: The External Alarm     or Video Loss     graphic is displayed on the monitor only when the alarm is activated.

7.9.5  Press “ESC” to confirm and return to main item selection.

7.9.6  Press “ESC” to exit on-screen menu.

7.10  SECURITY LOCK

It controls the multiplexer front panel security lock. When this feature is on, the multiplexer locks all front panel buttons except those required to re-enter the setup mode.

7.10.1  Press “ENTER+ ESC” simultaneously to enter the menu system setup

7.10.2  Press “▼” to move arrow down to SECURITY LOCK.

7.10.3  Press “ENTER” to enter system setup of SECURITY LOCK

7.10.4  Press “◄ or ►” to set external alarm ON ( shown on the bottom side of

time) or OFF

ON : all front panel button will be locked and only can be released

by press “ENTER + ESC” to setup MENU again.

OFF : no security lock feature.

    - Press “ESC” to confirm and return to main item selection.

    - Press “ESC” to exit on-screen menu.

7.11  SET TIME: to set up date and time

7.11.1  Press “ENTER+ ESC” simultaneously to enter the menu system setup

7.11.2  Press “▼” to move arrow down to SET TIME.

7.11.3  Press “ENTER” to enter system setup of SET TIME.

7.11.4  Press “▲ or▼” to select time display method as below

7.11.4.1 00/01/01 00:00:00 ( Year/Month/Day Hour: Minute: Second )

7.11.4.2 00/01/01 00:00:00 ( Day/Month/Year Hour: Minute: Second )

7.11.4.3 01/00/01 00:00:00 ( Month/Day/Year Hour: Minute: Second )

7.11.4.4 Time Display Off: No timing display on screen

7.11.5  Press       or       to set current time

7.11.6  Press “◄ or ►” to move left or right for setting time

7.11.7  Press “ESC” to confirm and return to main item selection.

7.11.8  Press “ESC” to exit on-screen menu.

7.12  EVENT LIST: to check the record of video loss or alarm status

7.12.1  Press “ENTER+ ESC” simultaneously to enter the menu system setup

7.12.2  Press “▼” to move arrow down to EVENT LIST.

---15 VLOSS 01/15/31 13:13:12

--- 8 ALARM 01/10/31 13:13:12

7.12.3  Press “ENTER” to enter system setup of EVENT LIST.

7.12.4  Press “◄ or ►” to set EVENT LIST

7.12.5  Press “ESC” to confirm and return to main item selection

7.12.6  Press “ESC” to exit on-screen menu.

7.13 BAUD RATE: to set up remote control baud rate

7.13.1  Press “ENTER+ ESC” simultaneously to enter the menu system setup

7.13.2  Press “▼” to move arrow down to REMOTE CONTROL BAUD RATE.

7.13.3  Press “ENTER” to enter system setup of REMOTE CONTROL BAUD RATE.

7.13.4  Press       or       to setup the Baud Rate (bps) for : 115200、57600、

19200、9600、4800、3600、2400、1200.

    - Press “ESC” to confirm and return to main item selection

7.13.6  Press “ESC” to exit on-screen menu.

7.14 ID

7.14.1  Press “ENTER+ ESC” simultaneously to enter the menu system setup

7.14.2  Press “▼” to move arrow down to ID.

7.14.3  Press “ENTER” to enter system setup of ID.

7.14.4  Press       or       to setup the ID number: 000~063.

7.14.5  Press “ESC” to confirm and return to main item selection.

7.14.6  Press “ESC” to exit on-screen menu.

7.15  REMOTE INTERFACE : RS485 and RS232C can be selected for remote control

interface

7.16  SYSTEM RESET

7.16.1  Press “ENTER+ ESC” simultaneously to enter the menu system setup

7.16.2  Press “▼” to move arrow down to SYSTEM RESET.

7.16.3  Press “ENTER” to enter SYSTEM RESET.

7.16.4  Press “ESC” to exit on-screen menu.

8. CHANNEL SETUP

Press “ENTER+ ESC” simultaneously to enter the menu set up, as shown in Figure 1, then press         to enter channel setup, as shown in Figure 2 (Menu will always display on Main monitor).  Press         to return to previous system setup.

Figure 2   CHANNEL SETUP

|  | TITLE |  | DWELL |  |  |  | ALARM | RECORD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ► | ---- | 01 | ON | 05 | 05 | 05 | LOW | EVENT |
|  | ---- | 02 | ON | 05 | 05 | 05 | LOW | EVENT |
|  | ---- | 03 | ON | 05 | 05 | 05 | LOW | EVENT |
|  | ---- | 04 | ON | 05 | 05 | 05 | LOW | EVENT |
|  | ---- | 05 | ON | 05 | 05 | 05 | LOW | EVENT |
|  | ---- | 06 | ON | 05 | 05 | 05 | LOW | EVENT |
|  | ---- | 07 | ON | 05 | 05 | 05 | LOW | EVENT |
|  | ---- | 08 | ON | 05 | 05 | 05 | LOW | EVENT |
|  | ---- | 09 | ON | 05 | 05 | 05 | LOW | EVENT |
|  | ---- | 10 | ON | 05 | 05 | 05 | LOW | EVENT |
|  | ---- | 11 | ON | 05 | 05 | 05 | LOW | EVENT |
|  | ---- | 12 | ON | 05 | 05 | 05 | LOW | EVENT |
|  | ---- | 13 | ON | 05 | 05 | 05 | LOW | EVENT |
|  | ---- | 14 | ON | 05 | 05 | 05 | LOW | EVENT |
|  | ---- | 15 | ON | 05 | 05 | 05 | LOW | EVENT |
|  | ---- | 16 | ON | 05 | 05 | 05 | LOW | EVENT |

  - TITLE

It allows you to assign a title to each camera input. Initially each title is the camera’s number.

    - Press “▲ or▼” : Select channel in the Channel setup

    - Press ENTER: Confirm the channel

    - Press “◄ or ►”: Select the location of title (6 characters maximum)

    - Press      or      : Select the characters for display (0~9, A~Z)

    - Press “ESC”:  Confirm and exit the title setting

    - Press “ESC”:  Exit the channel setup

8.2  DWELL: to set up auto switching channel

8.2.1  Press “▲ or▼” : Select channel in the Channel setup

8.2.2  Press “ENTER”:  Enter the channel setup

8.2.3  Press “◄ or ►”: Select DWELL on the screen

8.2.4  Press        or       : Select ON/OFF on the screen

8.2.5  Press “ESC”:  Confirm and exit the dwell setting

8.2.6  Press “ESC”:  Exit the channel setup

8.3   (Brightness) /  (Contrast) /  (Color): video adjustment of each channel

8.3.1  Press “▲ or▼” : Select channel in the Channel setup

8.3.2  Press “ENTER” Enter the channel setup

8.3.3  Press “◄ or ►”: Select brightness/contrast/color on the screen

8.3.4  Press      or     : Set up 0~9 on the screen.

8.3.5  Press “ESC”:  Confirm and exit the brightness/contrast/color setting.

8.3.6  Press “ESC”:  Exit the channel setup

8.4  ALARM

8.4.1  Press “▲ or▼” : Select channel in the Channel setup

8.4.2  Press “ENTER”  Enter the cannel setup

8.4.3  Press “◄ or ►”: Select ALARM on the screen

8.4.4  Press      or      : Select LOW/OFF/HIGH for alarm polarity

8.4.5  Press “ESC”:  Confirm and exit the alarm setting.

8.4.6  Press “ESC”:  Exit the channel setup

8.5  RECORD

8.5.1  Press “▲ or▼” : Select channel in the Channel setup

8.5.2  Press “ENTER”:  Enter the channel setup

8.5.3  Press “◄ or ►”: Select Record on the screen.

8.5.4  Press       or      : Select VCR record method as below (VCR will only record the channel which has been set up for EVENT/NORMAL/OFF)

8.5.4.1 EVENT: when alarm input is triggered, VCR will record alarming channel more
 			frequently. For example : VCR normal record method is 1-2-3-4-5-……-16. In 
 			Event mode, when CH01 is triggered, the record method will become
 			1-2-1-3-1-4-1-5-…-16.

8.5.4.2 NORMAL: when external alarm is on, VCR will record normally as set up.

8.5.4.3 OFF: VCR will not record

8.5.5  Press “ESC”:  Confirm and exit the record setting

8.5.6  Press “ESC”:  Exit the channel setup

9. MOTION DETECTION SETUP

9.1 Press “ENTER+ ESC” simultaneously to enter the menu set up, as shown in Figure 1, then press         to enter channel setup, as shown in Figure 2 (Menu will always display on Main monitor).

9.2 To access the motion setup screen, press ENTER then press Motion while the Camera title menu is displayed.  Each screen displays the current camera picture overlaid with the motion targets (as shown in Figure 3). You can move the arrows “◄ or ►, ▲ or▼” to adjust whether motion detection is ON or OFF.

9.3 The targets on each motion setup can be turned ON or OFF individually, by row or by screen. To set up targets, use the following front panel buttons:

9.3.1  Press ‘ENTER” to confirm the channel

9.3.2  Press “MOTION” to enter motion mode

9.3.3  ▲ Up arrow -- moves the target cursor up one row at a time.

9.3.4  ▼ Down arrow -- moves the target cursor down one row at a time.

9.3.5  ◄ Left arrow -- moves the target cursor left one column at a time.

9.3.6  ► Right arrow -- moves the target cursor right one column at a time.

9.3.7  Press “MOTION” to turn the target cursor ON

Press “MOTION” again to turn the target custor OFF

9.3.8  Camera Select (1-16) -– toggles the corresponding target on the cursor line ON or OFF.  (as shown in Figure 3-1) There are only 15 targets in one row, therefore only Camera Select (1-15) can be operated.

9.3.9  Line – turns all targets in the current row ON or OFF. (as shown in Figure 3-2)

9.3.10  All – turns all targets on the screen ON or OFF. (as shown in Figure 3-3)

9.4 To adjust       or       button to setup the Sensitivity list 000~255

Sensitivity value is related to motion and brightness change.

Low value (as 001) means higher sensitivity on motion and brightness change.

High value (as 255) means lower sensitivity on motion and brightness change.

User can choose the suitable sensitivity value in different locations. The default value is set on 32.

Figure 3   MOTION DETECTION SETUP

| 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  | 11 |  | 12 |  | 13 |  | 14 |  | 15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 032 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |

Figure 3-1   MOTION DETECTION SETUP – 1~16

| 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  | 11 |  | 12 |  | 13 |  | 14 |  | 15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 032 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |

Figure 3-2   MOTION DETECTION SETUP – LINE

| 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  | 11 |  | 12 |  | 13 |  | 14 |  | 15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 032 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |

Figure 3-3   MOTION DETECTION SETUP – ALL

| 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  | 11 |  | 12 |  | 13 |  | 14 |  | 15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 032 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |

- Monitor Operation

10.1  To connect with one monitor :

Please connect with MONITOR connector.

10.2  To connect with two monitors:

One monitor connects to “MONITOR” connector and shows live display / VCR playback. The other monitor connects to “CALL” connector to shows “auto switch” and “alarming channel” display.

10.3  Call Monitor display:

10.3.1  Call Monitor will switch 16 channels automatically in general.

10.3.2  The alarming channel will display full screen on the call monitor. While there are more than 2 channel alarms activated at the same time, the call monitor will auto switch to display for the above 2 channels in turns, in full screen only.

- Auto Switching

The multiplexer allows cameras to display sequentially on the main monitor in various

Formats available. In the full screen format, all cameras can be displayed sequentially.    In multi-screen formats, cameras which are not displayed on screen can be displayed sequentially in the lower right window or in the confirmed window.

To select the desired format

(Full Screen, or      or      or      or       or      or ZOOM)

11.1 Auto switching of Full screen

11.1.1 The monitor screen under full screen display

11.1.2 Press “ENTER” button to start auto switching

11.1.3 Press “ENTER” button to stop auto switching

11.2 Auto switching of 4, 7, 9, 10, 13, 16 channels display mode

11.2.1 The monitor screen under the above display mode.

11.2.2 Press “ENTER” button to start auto switching.

At the lower-right window of the display screen, Auto Switching displays only  the channels which do not display on the monitor screen.

11.2.3  Press “ENTER” button to stop auto switching

11.2.4  Press “ ENTER + Camera Select (1-16) “ to start the auto switching at the appointed window. The appointed window means the window location (in the sequential order from left to right and top to bottom) displayed on monitor screen. It only displays the auto switching of the channels which do not display on the monitor screen.

11.3 Auto switching of ZOOM mode

11.3.1 The monitor screen under the ZOOM display mode

11.3.2 Press “ENTER” button to start auto switching

11.3.3 Press “ ENTER” button to stop auto switching

11.4 Auto switching of PIP mode

11.4.1 The monitor screen under the PIP display mode

11.4.2 Press “ENTER” button to start auto switching of the small insert window

11.4.3 Press “ ENTER” button to stop auto switching

11.4.4. Press “ ENTER + Camera channel # on main screen ” to start the auto

switching on main screen. It only displays the auto switching

of the channels which do not display on the monitor screen.

11.5 Auto switching of QUAD DISPLAY mode       

11.5.1 The monitor screen under        mode

11.5.2 Press “ENTER +        ” button to start auto switching of QUAD DISPLAY

mode

11.5.3 Press “ENTER” button to stop auto switching

- VCR Playback Operation

To review a videotape with the multiplexer, you need to press PLAY button of VCR. The multiplexer decodes the recorder video signal and displays it in the current format, showing  on the bottom the of the channel.  Cameras appear in the order they were recorded.

Multiplexer operation during videotape playback is the same as when viewing live cameras.  All front panel buttons have the same effect.  The difference is that VCR is the source of video. Please refer to section 5.6 for details of VCR playback operation

- External Alarm Input and Video Loss Signal, when activated

01

13.1 External alarm input

Live Display Screen

    - Live display screen: alarming channel will show  on the right of the channel
title. If the alarming channel is not currently displayed on the screen, the
monitor will jump to 16 channels display mode while alarm is activated.

    - Auto switch display screen: screen will display alarming channel. When there
are more than 2 alarming channels , monitor will auto switch to show the
alarming channels in turns.

13.1.3  VCR RECORD time will auto reset to alarm VCR record time, and record more
 frequently for alarming channels.

13.2 Video loss

This multiplexer has 2 visual ways of letting you know when the video losses it’s signal,

    - is shown on the left side of channel title. If the video loss channel is not currently displayed on the screen, the screen will jump to 16 channels display mode while video is activated.

- VCR Time Setting

14.1  Multiplexer’s VCR record time setting should be the same as VCR record time setting. If AUTO is used, multiplexer will automatically set it’s own VCR time setting to be the same as VCR time setting.

  - Auto setting: If the VCR has “VCR Trigger Out” control signal connected with multiplexer’s record trigger input, multiplexer will auto detect the VCR timer setting. If not, Multiplexer will follow it’s own original setting.

14.3  Manual setting : If VCR does not have “VCR Trigger Out” control signal, the multiplexer needs to set up VCR record time on the screen menu. Because different VCRs have different settings, please refer to Record Time Flash Rate Chart for reference.

15.  Record Time Flash Rate Chart:

NTSC System:

| Record Mode | VIDEO INPUT QUANTITY |  |  |
| --- | --- | --- | --- |
|  | 1 CHANNEL (Picture/second) | - CHANNEL (Picture/second) | 16 CHANNEL (Picture/second) |
| 002H | ~ 15.000 | ~ 3.750 | ~ 0.938 |
| 006H | ~ 10.000 | ~ 2.500 | ~ 0.625 |
| 008H | ~ 7.500 | ~ 1.875 | ~ 0.469 |
| 012H | ~ 5.000 | ~ 1.250 | ~ 0.313 |
| 015H | ~ 4.000 | ~ 1.000 | ~ 0.250 |
| 016H | ~ 3.750 | ~ 0.938 | ~ 0.235 |
| 018H | ~ 3.333 | ~ 0.833 | ~ 0.208 |
| 020H | ~ 3.000 | ~ 0.750 | ~ 0.188 |
| 024H | ~ 2.500 | ~ 0.625 | ~ 0.156 |
| 036H | ~ 1.667 | ~ 0.417 | ~ 0.104 |
| 040H | ~ 1.500 | ~ 0.375 | ~ 0.094 |
| 048H | ~ 1.250 | ~ 0.313 | ~ 0.078 |
| 060H | ~ 1.000 | ~ 0.250 | ~ 0.063 |
| 072H | ~ 0.833 | ~ 0.208 | ~ 0.052 |
| 080H | ~ 0.750 | ~ 0.188 | ~ 0.047 |
| 084H | ~ 0.667 | ~ 0.167 | ~ 0.042 |
| 120H | ~ 0.500 | ~ 0.125 | ~ 0.031 |
| 160H | ~ 0.375 | ~ 0.094 | ~ 0.024 |
| 168H | ~ 0.333 | ~ 0.083 | ~ 0.021 |
| 240H | ~ 0.250 | ~ 0.063 | ~ 0.016 |
| 360H | ~ 0.167 | ~ 0.042 | ~ 0.011 |
| 480H | ~ 0.125 | ~ 0.031 | ~ 0.008 |
| 720H | ~ 0.083 | ~ 0.021 | ~ 0.005 |
| 960H | ~ 0.063 | ~ 0.013 | ~ 0.003 |

PAL System:

| RECORD MODE | VIDEO INPUT QUANTITY |  |  |
| --- | --- | --- | --- |
|  | 1 CHANNEL | 4 CHANNEL | 16 CHANNEL |
| 003H | ~ 12.500 | ~ 3.125 | ~ 0.781 |
| 006H | ~ 8.333 | ~ 2.081 | ~ 0.520 |
| 012H | ~ 6.250 | ~ 1.563 | ~ 0.391 |
| 015H | ~ 5.000 | ~ 1.250 | ~ 0.313 |
| 018H | ~ 4.167 | ~ 1.042 | ~ 0.261 |
| 024H | ~ 3.125 | ~ 0.781 | ~ 0.195 |
| 030H | ~ 2.500 | ~ 0.625 | ~ 0.156 |
| 036H | ~ 2.083 | ~ 0.521 | ~ 0.130 |
| 048H | ~ 1.563 | ~ 0.391 | ~ 0.098 |
| 060H | ~ 1.250 | ~ 0.313 | ~ 0.078 |
| 072H | ~ 1.042 | ~ 0.261 | ~ 0.065 |
| 090H | ~ 0.833 | ~ 0.208 | ~ 0.052 |
| 120H | ~ 0.625 | ~ 0.156 | ~ 0.039 |
| 168H | ~ 0.417 | ~ 0.104 | ~ 0.026 |
| 240H | ~ 0.313 | ~ 0.078 | ~ 0.020 |
| 480H | ~ 0.156 | ~ 0.039 | ~ 0.010 |
| 720H | ~ 0.104 | ~ 0.026 | ~ 0.007 |
| 960H | ~ 0.078 | ~ 0.020 | ~ 0.005 |

16.  Remote Control Function

DATA: 1 Start bit ,8bit data ,none,1 Stop bit

ACT       			COH     					ID         FUNCTION       STOP

(FFH)																													 (7FH)

| FUNCTION | CODE | ASCII | FUNCTION | CODE | ASCII |
| --- | --- | --- | --- | --- | --- |
| MENU | 0x4D | M | 14 | 0x45 | E |
| ESC | 0x1B | ESC | 15 | 0x46 | F |
| FREEZE | 0x5A | Z | 16 | 0x47 | G |
| LIVE | 0x49 | I | ENTER+FREEZE | 0x48 | H |
| VCR | 0x56 | V | ENTER+LIVE | 0x4A | J |
| ZOOM | 0x7A | z | ENTER+VCR | 0x4F | O |
| PIP | 0x50 | P | ENTER | 0x0D | ENTER |
| LEFT | 0x4C | L | SEQUENCE 1 | 0x61 | a |
| RIGHT | 0x52 | R | SEQUENCE 2 | 0x62 | b |
| UP | 0x55 | U | SEQUENCE 3 | 0x63 | c |
| DOWN | 0x4E | N | SEQUENCE 4 | 0x64 | d |
| KEY_LOCK | 0x4B | K | SEQUENCE 5 | 0x65 | e |
| 1 | 0x31 | 1 | SEQUENCE 6 | 0x66 | f |
| 2 | 0x32 | 2 | SEQUENCE 7 | 0x67 | g |
| 3 | 0x33 | 3 | SEQUENCE 8 | 0x68 | h |
| 4 | 0x34 | 4 | SEQUENCE 9 | 0x69 | i |
| 5 | 0x35 | 5 | SEQUENCE 10 | 0x6A | j |
| 6 | 0x36 | 6 | SEQUENCE 11 | 0x6B | k |
| 7 | 0x37 | 7 | SEQUENCE 12 | 0x6C | l |
| 8 | 0x38 | 8 | SEQUENCE 13 | 0x6D | m |
| 9 | 0x39 | 9 | SEQUENCE 14 | 0x6E | n |
| 10 | 0x41 | A | SEQUENCE 15 | 0x6F | o |
| 11 | 0x42 | B | SEQUENCE 15 | 0x6F | o |
| 12 | 0x43 | C | SEQUENCE 16 | 0x70 | p |
| 13 | 0x44 | D |  |  |  |

17. Connection:

Basic Connection for Multiplexer – with 1 monitor

Full Duplex Connections for Multiplexer – with 2 monitors

18. Rack Mount Installation:

Ear

Front Panel

Ear

Rack Mount

Front Angle with Rack Mount

Ear

Side Angle View