Digital Multiplex Recorder

User Manual

777 V 1.1

# Table of Contents

Introduction

3

4

5

6

6

7

8

9

11

13

19

19

25

27

29

31

33

36

37

37

38

38

39

39

39

40

40

41

42

  - Safety Warning  …………………………………………………………………………………

  - Features  ………………………………………………………………………………………….

1-3 Specifications  …………………………………………………………………………………...

- Installation

2-1 Package contents  ……………………………………………………………………………….

2-2 Connection   ………………………………………………………………………………….….

2-3 Rack mount installation  …………………………………………………………….…………..

2-4 RS232 /RS485 Remote protocol ……………………………………………………………..…

- Configuration

3-1 Install HDD  …………………………………………………………………………………….…

3-2 Front panel keypad  ……………………………………………………………………………..

3-3 Back panel connection  ………………………………………………………………………….

3-4 Menu setup  ……………………………………………………………………………………….

3-5 System setup  …………………………………………………………………………………….

3-6 Search setup  … ………………………………………………………………………………....

3-7 TIMER setup (Schedule Time-Lapse record mode setup )………………………………….…

3-8 Record Setup   …………………………………………………..………………………….…. ..

3-9 Camera setup  ……….……………………………………………………………………….….

3-10 Motion Detection setup   ……………..…………………………………………………….…

3-11 Event   ……………………………………………………………………………………….…..

- Operation

4-1 Power on  …………………………………………………………………………………………

4-2 Recording  ………………………………………………………………………………………...

4-3 Camera Select (1-16) ………... ……………………………………………………………….….

4-4 Select……. ………………………………………………………………………………………...

4-5 Picture in Picture………..…………………………..……………………………………………...

4-6 Zoom  ………………………………………………………………………………………………

4-7 Play   ……………………………………………………………………………………………….

4-8 Video Loss  ………………………………………………………………………………………..

4-9 Key Lock …………………………………………………………………………………………….

- Trouble shooting & appendix

5-1 Trouble shooting  …………………………………………………………………………………

5-2 Compatible HDD brands  …………………………………………………………………...…...

1. Introduction

Thank you for choosing this high quality Digital Multiplex Recorder. The DMR converts analog NTSC or PAL video to digital images and records them to a removable hard disk drive. Digitally recorded video has many advantages compared to analog video recorded on tape. There is no need to adjust tracking. Digital video can be indexed by time schedule or events, and you can instantly view video after selecting the time or event. You can freeze frames, fast-forward, fast- reverse, slow-forward, and slow-reverse without image streaking or tearing. It can be used as a replacement for both a time-lapse VCR and a multiplexer in a security installation.

  - Safety Warning

All the safety and operating instructions should be read before the operation. The improper operation may cause permanent damage.

1.1.1 Please use the provided adaptor.

1.1.2 Please lift and place this equipment gently.

1.1.3 Do not expose this equipment to open sunlight.

1.1.4 Do not use this equipment near water or in contact with water.

1.1.5 Do not spill liquid of any kind on the equipment.

1.1.6 Please power down the unit before unplugging.

1.1.7 This equipment should be operated only with power source from standard package.

1.1.8 Unauthorized repair or parts substitutions may result in fire, electric shock or other     hazards.

1.1.9  Do not switch the Power On & Off within short period o time (within 3 seconds).

1.1.10 Do not attempt to service this equipment by yourself. Refer all servicing and repair to qualified service personnel.

1.1.11 This unit should be operated only from power source indicated on the manufacturer’s label.

1.1.12 Installation should be made by qualified service personnel and should conform to all local safety codes and regulations.

1.2  Features

1.2.1 Wavelet Compression Format replaces Time-Lapse VCR + Multiplexer

1.2.2 4 Audio inputs / 2Audio outputs

1.2.3 On Screen Display and RTC (Real time clock) Function

1.2.4 Multiplexing

- Support from 4 channels up to 7/ 9/ 10/ 13/ 16 channels.

- 16 channels can record or display real-time recorded images.

- 16 channels loop back terminal.

    - Resolution

| Screen Modes | ###### Resolution |
| --- | --- |
| Full screen | 704(H) x 468(V) <NTSC> / 704(H) x 564(V) <PAL> |
| 4 channels | 352(H) x 234(V) <NTSC> / 352(H) x 282(V) <PAL> |
| 9 channels | 224(H) x 156(V) <NTSC> / 224(H) x 188(V) <PAL> |
| 16 channels | 176(H) x 117(V) <NTSC> / 176(H) x 141(V) <PAL> |

    - Independent main and call monitor outputs allow simultaneous multi-camera or       full screen viewing.

    - Multi Screen Display Mode

- Below display modes are selectable while surveillance & DMR play back:

    - Picture-In-Picture (PIP) is available in live and DMR playback modes, Zoom capability up to 2 X 2 in live and DMR playback modes

    - 16 channels are swappable and each channel has independent title generator (up to 6 characters).

    - Motion detection using 15x12 target (NTSC) 15x14 target (PAL) motion detection grids for each video channel.

    - Video Quality Adjustable on Each Channel

    - Alarm Input Function & Output Function

    - 16 channels alarm input, ALARM display and one alarm output.

- Video loss detected on each channel can record 160 events.

    - Power-loss memory function: set up parameter will remain, in case of power failure.

    - Call Monitor: Switch Display

    - Support 2 Removable HDD, IDE Type

    - Timer: Schedule recording

    - Display refresh rate up to 72 IPS (60 IPS for PAL)

    - Record refresh rate up to 15 IPS (12 IPS for PAL)

    - Quick Multiple Search by date/time, alarm, full list

    - Fast and slow playback in multiple speeds.

    - Security password protection

    - RS-232, RS-485 communication protocol

  - Specifications

| Video format | NTSC/EIA or PAL/CCIR |
| --- | --- |
| Hard disk storage | IDE type, UTMA 66 above, 2 removable HDDs supported |
| Record mode | Manual / Alarm / Timer |
| Camera Input Signal | Composite video signal 1 Vp-p 75Ω BNC, 16 channels |
| # Camera Loop Back | Composite video signal 1 Vp-p 75Ω BNC, 16 channels |
| Main Monitor Output | Composite video signal 1 Vp-p 75Ω BNC |
| Call Monitor Output | Composite video signal 1 Vp-p 75Ω BNC |
| Audio input | 4 audio inputs, (RCA) |
| Audio output | 2 audio outputs, (RCA) |
| Motion Detect Area | 15 * 12 targets per camera (NTSC) / 15 * 14 targets per camera (PAL) |
| # Motion Detect Sensitivity | 256 Levels |
| # Video Loss Detection | Yes |
| # Refresh Rate | 72 images/sec. for NTSC / 60 images/sec. for PAL |
| # Recording Rate | 15 images/sec. for NTSC / 12 images/sec. for PAL |
| # Dwell Time | Programmable (1~10 Sec) |
| # Picture in Picture | Yes (Movable) |
| # Key Lock | Yes |
| # Picture Zoom | 2*2 (Movable) |
| # Camera Title | 6 letters |
| # Video Adjustable | Color/ Contrast/ Brightness Adjustable |
| ## Alarm Input | TTL input, Hi (5V), Low (GND) |
| ## Alarm Output | COM,/N.O |
| ## Remote Control | RS-232 or RS-485 |
| Time Display Format | YY/MM/DD, DD/MM/YY, MM/DD/YY, OFF |
| Power Source | AC100~240V + 10% switching adaptor |
| Power Consumption | <45W |
| Operation Temperature | 10 ~ 40℃ |
| RS-232C / RS-485 (bps) | 115200、57600、19200、9600、4800、3600、2400、1200 |
| Dimension (mm) | 432(W) x 110(H) x 325(D) |
| Net Weight | 5.7 kgs |

Specifications are subject to change without notice.

2. Installation

  - Package contents

The package contents the following items.

- Digital Multiplex Recorder

- HDD cartridge

- Key for cartridge (Inside of HDD cartridge)

- Power cord

- User manual

- Rack mounting kit (Optional)

  - Connection

2.2.1 Connect with cameras

  - Rack mount installation

Front Angle with Rock Mount

Side View with Rack Mount

  - RS232 Remote Protocol

You can use the PC keyboard to simulate DMR keypad.

DATA: REMOTE PROTOCOL using 8 bit data、1 start bit、1stop bit

ACT       			C0H     					ID         FUNCTION       STOP

(FFH)																													 (7FH)

| FUNCTION | CODE | ASCII | FUNCTION | CODE | ASCII |
| --- | --- | --- | --- | --- | --- |
| KEY_MENU | 0x4D | M | KEY_CH1 | 0x31 | 1 |
| KEY_SELECT | 0x73 | s | KEY_CH2 | 0x32 | 2 |
| KEY_ENTER | 0x0D | ENTER | KEY_CH3 | 0x33 | 3 |
| KEY_4CUT | 0x61 | a | KEY_CH4 | 0x34 | 4 |
| KEY_ZOOM | 0x5A | Z | KEY_CH5 | 0x35 | 5 |
| KEY_9CUT | 0x62 | b | KEY_CH6 | 0x36 | 6 |
| KEY_PIP | 0x70 | p | KEY_CH7 | 0x37 | 7 |
| KEY_16CUT | 0x63 | c | KEY_CH8 | 0x38 | 8 |
| KEY_SLOW | 0x53 | S | KEY_CH9 | 0x39 | 9 |
| KEY_REC | 0x72 | r | KEY_CH10 | 0x41 | A |
| KEY_LEFT | 0x4C | L | KEY_CH11 | 0x42 | B |
| KEY_UP | 0x55 | U | KEY_CH12 | 0x43 | C |
| KEY_PLAY | 0x50 | P | KEY_CH13 | 0x44 | D |
| KEY_DOWN | 0x4E | N | KEY_CH14 | 0x45 | E |
| KEY_RIGHT | 0x52 | R | KEY_CH15 | 0x46 | F |
| KEY_POWER | 0x57 | W | KEY_CH16 | 0x47 | G |
| KEY_KEY_LOCK | 0x4B | K |  |  |  |

3. Configuration

  - Install HDD

- Installing Hard Drive into Cartridge - Please find the Key in the Cartridge. - Push the front cover of the HDD cartridge and use the miniature key provided and insert into the keyhole, turning the key counter-clockwise. - Pull the handle outwards to remove the carrier body away from the cartridge frame. - Push the open button to slide the top cover backwards and remove. - Insert the DC power cable and IDE cable on the HDD. - Position the HDD into carrier body and secure the HDD with the four 6#-32 screws provided. - Slide the top cover back to the carrier body by sliding it forward to secure. - Slide the carrier body back into the cartridge frame.

3.1.2  Function Settings and Operation

3.1.2.1   Power Indicator and HDD Access indicator

When power is on, the indicator will display the following message：

| Item | Indicator |
| --- | --- |
| Power Indicator | Green LED |
| HDD Access Indicator | Yellow LED |

3.1.3  Key lock

| Status A B Segment | Power status | Security status |
| --- | --- | --- |
| A | ON | Locked (Irremovable) |
| B | OFF | Unlocked (Removable) |

You must turn key lock to “ A ” position before powering on the unit. Otherwise HDD will not operate properly.

3.1.4  If two HDDs are set up, one should be selected as “ Master “ and the other should be selected as “ Slave”. Otherwise, please set two HDDs to Cable Select.

3.1.1.1 This product do not support hot swap. Please power off the unit first before removing HDD.

3.1.1.2 The models and manufactures of HDD listed above have been tested and proven compatible with this appliance.

3.1.1.3 Please remove hard disk drive ONLY after power was shut off for more than 60 seconds. This would protect and extend the operating life of the hard disk drive.

3.1.1.4 Master or Slave HDD should be set up before HDD Jumper.

3.1.1.5 Cable Select is suggestive for HDD Jumper.

3.1.4 The Record Time is different based on Record Speed and Record Quality.                   Please refer to following table.

NTSC SYSTEM

| IPS |  | 15A | 15 | 8 | 4 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record Quality | Best | 48hr | 96hr | 180hr | 360hr | 720hr | 1440hr |
|  | High | 60hr | 120hr | 226hr | 450hr | 900hr | 1800hr |
|  | Normal | 96hr | 192hr | 360hr | 720hr | 1440hr | 2880hr |
|  | Basic | 160hr | 320hr | 600hr | 1200hr | 2400hr | 4800hr |
| HDD Type |  | 240GB |  |  |  |  |  |

PAL SYSTEM

| IPS |  | 12A | 12 | 6 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record Quality | Best | 48hr | 100hr | 202hr | 406hr | 608hr | 1216hr |
|  | High | 60hr | 126hr | 254hr | 506hr | 760hr | 1520hr |
|  | Normal | 98hr | 202hr | 406hr | 810hr | 1216hr | 2440hr |
|  | Basic | 162hr | 336hr | 676hr | 1350hr | 2026hr | 4050hr |
| HDD Type |  | 240GB |  |  |  |  |  |

Note: Above data is from actual test data obtained from recording normal TV program. (Reference only)

  - Front panel keypad

Please follow the instructions below to operate this unit.

- Install HDD

This device equips with a removable hard disk drive tray. Before turning on this device, you must install hard disk drive (not including in standard package), and make it ready to operate.

(If two HDDs are set up, one should be selected as “ Master” and the other should be selected as “ Slave”. Otherwise, please set two HDDs to Cable Select.)

- MENU : Press MENU to enter main menu operation mode, and press administrator password (default:0000) to access main menu.

- ENTER : Press ENTER for confirmation.

- ZOOM : Press ZOOM button to enlarge the main monitor picture display.

- : Picture in Picture. Press PIP button to enter PIP display screen.

- SLOW : To press SLOW to slow down speed of play mode.

- Select : Press the Select to select appointed camera (1~16) to display on full screen mode.

- : 4 channels display mode

- : 7, 9, 10, 13 channels display mode

- : 16 channels display mode

- LED Light : Under following condition, the LED Light is ON.

(1) HDD : HDD is activated

(2) HDD Full：HDD is full

(3) ALARM：When Alarm Enable : Yes (when alarm is triggered, the led is flashing)

(4) TIMER：When Timer Enable : Yes

(5) PLAY：Play operation

(6) REC：Recording operation

- CAMERA (1-16) : Press the Camera Select (1-16) to select appointed camera

- REC : Press REC to start recording.

- REW / Left :

REW : Under DMR play mode, it can play video backward at high speed, and press REW again to adjust speed from 1, 2, 4, 8, 16, 32 times

Left : Under setup mode , it can work as Left button.

- PLAY : Press PLAY to play recorded video.

- STOP / Down :

STOP : Under DMR Record/Play mode, it can play video stop recording.

Down : Under setup mode , it can work as Down button.

- PAUSE / Up :

Pause : Punder DMR play mode, it can play video pause.

Up : Under setup mode , it can work as Up button.

- FF / Right :

FF : Under DMR play mode, it can play video forward at high speed, and press FF again to adjust speed from 1, 2, 4, 8, 16, 32 times.

Right : Under setup mode , it can work as Right button.

- POWER : To power on, press power button.

To power off, press power button again.

PS1 . After 60 seconds suspended operation, the function setup will be turned up automatically.

2. When setting the same channel, the duplicate pictures will be frozen.

3.3 Back panel connection

3.3.1 Power input

Please use the provided adaptor.

3.3.2 External I/O

Controlled remotely by an external device or control system.

Alarm input, external I/I explanation.

3.3.3 75 / HI

When usingLoop function, please switch to HI or set up to 75.

    - VIDEO INPUT (1-16)

Connect to video source, such as camera.

    - Loop

Connect video signal between Input port and Loop port to make a loop.

    - AUDIO IN (1-4)

Connect to audio source, such as microphone.

* IPS should be set to 15A (for NTSC) or 12A (for PAL)

    - AUDIO OUT (R/L)

Connect to monitor or speaker.

* IPS should be set to 15A (for NTSC) or 12A (for PAL)

    - MONITOR

Connect to Main monitor

    - CALL

Connect to CALL monitor.  Show the Switch Display.

3.3.9 External I/O

25 pin com port

9 pin com port

PIN 1.   GND

GROUND

PIN 2.  ALARM INPUT 8

To connect wire from ALARM INPUT 8 ( PIN 2 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on.

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 3. ALARM INPUT 6

To connect wire from ALARM INPUT 6 ( PIN 3 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on.

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 4.  ALARM INPUT 4

To connect wire from ALARM INPUT 4 ( PIN 4 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 5.  ALARM INPUT 2

To connect wire from ALARM INPUT 2 ( PIN 5 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on.

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 6. ALARM INPUT 16

To connect wire from ALARM INPUT 16 ( PIN 6 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on.

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 7.  ALARM INPUT 14

To connect wire from ALARM INPUT 14 ( PIN 7 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on.

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 8.  ALARM INPUT 12

To connect wire from ALARM INPUT 12 ( PIN 8 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on.

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 9.   ALARM INPUT 10

To connect wire from ALARM INPUT 10 ( PIN 9 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on.

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 10.  PIN OFF

PIN 11.  RS232-TX

DMR can be controlled remotely by an external device or control system, such as a control keyboard, using RS-232 serial communications signals.

PIN 12.  RS485-A

DMR can be controlled remotely by an external device or control system, such as a control keyboard, using RS485 serial communications signals.

PIN 13.  EXTERNAL ALARM NO

Under normal operation COM disconnect with NO. But when Alarm triggered, COM connect with NO.

PIN 14.  PIN OFF

PIN 15.  ALARM INPUT 7

To connect wire from ALARM INPUT 7 ( PIN 15 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on.

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 16.  ALARM INPUT 5

To connect wire from ALARM INPUT 5 ( PIN 16 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on.

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 17.  ALARM INPUT 3

To connect wire from ALARM INPUT 3 ( PIN 17 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on.

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 18. ALARM INPUT 1

To connect wire from ALARM INPUT 1 ( PIN 18 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on.

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 19. ALARM INPUT 15

To connect wire from ALARM INPUT 15 ( PIN 19 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on.

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 20.  ALARM INPUT 13

To connect wire from ALARM INPUT 13 ( PIN 20 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on.

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 21. ALARM INPUT 11

To connect wire from ALARM INPUT 11 ( PIN 21 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on.

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 22.  ALARM INPUT 9

To connect wire from ALARM INPUT 9 ( PIN 22 ) to GND ( PIN 1 ) connector, DMR will start recording and buzzer will be on.

When Menu/ Camera/ Alarm is set up to “Low” : When alarm input signal is “ Low ”,

the unit starts to record and buzzer.

When Menu/ Camera/ Alarm is set up to “High” : When alarm input signal is “ High ”,

the unit starts to record and buzzer.

PIN 23. RS232-RX

DMR can be controlled remotely by an external device or control system, such as a control keyboard, using RS-232 serial communications signals.

PIN 24. RS485-B

DMR can be controlled remotely by an external device or control system, such as a control keyboard, using RS485 serial communications signals.

PIN 25. EXTERNAL ALARM COM

Under normal operation COM disconnect with NO. But when alarm triggered, COM connect with NO.

3.4  Menu setup

| Press ”MENU” to enter main menu. You will need to enter password to access main menu. To press “Right” “Left” to move digit, and Press ”Up” “Down” to select number. Press ”ENTER” button to confirm password. Ex.: Password : 0000 (Default : 0000) | PASSWORD : 0000 |
| --- | --- |

| After keying in correct password, confirm by pressing ”ENTER” button, screen will show following options. SEARCH ------- Find recorded list TIMER -------- Scheduling Record RECORD------- Record Mode Setup CAMERA ------- Camera Channel Setup SYSTEM ------- System Setup EVENT -------- Event List | (MENU) ► SEARCH TIMER RECORD CAMERA SYSTEM EVENT |
| --- | --- |

*For the unit can be operated properly, we start to introduce from System setup

3.5 System setup

| Press ”MENU” to enter main menu. You will need to enter password to access main menu. Press “Right” “Left” to move digit, and Press ”Up” “Down” to select number. Press ”ENTER” button to confirm password. Ex.: PASSWORD : 0000 (Default : 0000) | PASSWORD : 0000 |
| --- | --- |

| After keying in correct password, and confirm by pressing ”ENTER” button, screen will show following options. SEARCH ------- Find recorded list TIMER -------- Scheduling Record RECORD------- Record Mode Setup CAMERA ------- Camera Channel Setup SYSTEM ------- System Setup EVENT -------- Event List | (MENU) ► SEARCH TIMER RECORD CAMERA SYSTEM EVENT |
| --- | --- |

| Press ”Up” “Down” to choose SYSTEM setup SEARCH TIMER RECORD CAMERA ►SYSTEM EVENT | (Menu) SEARCH TIMER RECORD CAMERA ►SYSTEM EVENT |
| --- | --- |

Press ”ENTER” to confirm SYSTEM setup, and the screen will show following options.

|  | (SYSTEM) |  |
| --- | --- | --- |
| ► | AUDIO INPUT : 1 |  |
|  | INT AUDIBLE ALARM : ON |  |
|  | EXT AUDIBLE ALARM : ON |  |
|  | ALARM DURATION : 10 SEC |  |
|  | DWELL TIME : 02 SEC |  |
|  | MESSAGE LATCH : NO |  |
|  | TITLE DISPLAY :ON |  |
|  | TIME DISPLAY : Y/M/D |  |
|  | 2003-JAN-02(THU) 17:37:09 |  |
|  | NEW PASSWORD : XXXX |  |
|  | CLEAR HDD : MASTER |  |
|  | SYSTEM RESET : NO |  |
|  | REMOTE MODE : RS-232 |  |
|  | BAUD RATE : 9600 |  |
|  | REMOTE ID : 000 |  |

3.5.1 AUDIO INPUT setup:

This device allows user to set the AUDIO INPUT.  User can choose one               of 4 channels to record.

3.5.1.1 Press ”Up” “Down”  to choose AUDIO INPUT :

3.5.1.2 Press ”ENTER” button to confirm AUDIO INPUT

3.5.1.3 Press ”Up” “Down” to choose the AUDIO INPUT : 1~4

3.5.1.4 Press ”MENU” to exit and confirm current operation.

3.5.1.5 Press ”MENU” again to exit and close SYSTEM setup mode.

3.5.2 INT AUDIBLE ALARM setup:

This device allows user to set the INTERNAL AUDIBLE ALARM. Alarm will be trigged by event occurrence when the setting is ON.

3.5.2.1 Press ” Up” or “ Down” to choose INT AUDIBLE ALARM

3.5.2.2 Press ” ENTER” button to confirm INT AUDIBLE ALARM

3.5.2.3 Press ” Up” or “ Down” to choose the INT AUDIBLE ALARM : ON/OFF

ON: Internal Buzzer ON

OFF: Internal Buzzer OFF

3.5.2.4 Press ” MENU” to exit and confirm current operation.

3.5.2.5 Press ” MENU ” again to exit and close SYSTEM setup mode.

3.5.3 EXT AUDIBLE ALARM setup:

This device allows user to set the EXTERNAL AUDIBLE ALARM. Alarm will be trigged by event occurrence when the setting is ON.

3.5.3.1 Press ” Up” or “ Down” to choose EXT AUDIBLE ALARM

3.5.3.2 Press ” ENTER” button to confirm EXT AUDIBLE ALARM

3.5.3.3 Press ” Up” or “ Down” to choose the EXT AUDIBLE ALARM: ON/OFF

ON: External Buzzer ON

OFF: External Buzzer OFF

3.5.3.4 Press ” MENU” to exit and confirm current operation.

3.5.3.5 Press ” MENU” again to exit and close SYSTEM setup mode.

3.5.4 ALARM DURATION Setup option:

This device allows user to set the ALARM DURATION. Reaction time was determined by how long the alarm mode responded to a buzzer.

3.5.4.1 Press ” Up” or “ Down” to choose ALARM DURATION setup.

3.5.4.2 Press ” ENTER” to confirm ALARM DURATION setup.

3.5.4.3 Press ” Up” or “Down” to choose ALARM DURATION setup

10 SEC, 15 SEC, 20 SEC, 30 SEC, 1MIN, 2MIN, 3 MIN, 5 MIN, 10 MIN, 15 MIN, 30 MIN, ALWAYS.

3.5.4.4 Press ” MENU” to exit and confirm current operation.

3.5.4.5 Press ” MENU” again to exit and close SYSTEM setup mode.

3.5.5 DWELL TIME setup

This device allows user to set the DWELL TIME. DWELL TIME is the lasting time that each channel sequentially shown on monitor.

3.5.5.1 Press ” Up” or “ Down” to choose DWELL TIME setup.

3.5.5.2 Press ” ENTER” to confirm DWELL TIME setup.

3.5.5.3 Press ” Up” or “ Down” to choose DWELL TIME setup.

01~10 SEC.

3.5.5.4 Press ” MENU” to exit and confirm current operation.

3.5.5.5 Press ” MENU” again to exit and close SYSTEM setup mode.

3.5.6 MESSAGE LATCH setup

This device allows user to decide the   External Alarm graph    video loss graph showed on monitor or not.

3.5.6.1 Press ” Up” or “ Down” to choose MESSAGE LATCH setup

3.5.6.2 Press ” ENTER” button to confirm MESSAGE LATCH setup

3.6.6.3 Press ” Up” or “ Down” to choose the MESSAGE LATCH : ON/OFF

ON : Message latch ON

OFF : Message latch OFF

3.6.6.4 Press ” MENU” to exit and confirm current operation.

3.6.6.5 Press ” MENU” again to exit and close SYSTEM setup mode.

3.5.7 TITLE DISPLAY setup

This device allows user to set the title shown on monitor or not.

3.6.7.1 Press ” Up” or “ Down ” to choose TITLE DISPLAY setup

3.6.7.2 Press ” ENTER” button to confirm TITLE DISPLAY setup

3.6.7.3 Press ” Up” or “ Down ” to choose TITLE DISPLAY : ON/OFF

ON : Title Display ON

OFF : Title Display OFF

3.6.7.4 Press ” MENU ” to exit and confirm current operation.

3.6.7.5 Press ” MENU ” again to exit and close SYSTEM setup mode.

3.5.8 TIME DISPLAY format setup

This device allows user to set the time format on monitor.

3.5.8.1 Press ” Up” or “ Down ” to choose TIME DISPLAY setup.

3.5.8.2 Press ” ENTER ” to confirm TIME DISPLAY setup.

3.5.8.3 Press” Up ” or “ Down ” to choose TIME DISPLAY setup.

OFF, Y/M/D, M/D/Y, D/M/Y.

3.5.8.4 Press ” MENU ” to exit and confirm current operation.

3.5.8.5 Press ” MENU ” again to exit and close SYSTEM setup mode.

3.5.9 TIME setup:

This device allows user to set time on monitor.

3.5.9.1 Press ” Up ” or “ Down ” to choose date display

3.5.9.2 Press ” ENTER ” to confirm date display format.

3.5.9.3 Press ” Up ” or “ Down ” to choose the correct Date & Time, and press press “ Right ” or “ Left ” to move digit location.

3.5.9.4 Press ” MENU ” to exit and confirm current operation.

3.5.9.5 Press ” MENU ” again to exit and close SYSTEM setup mode.

3.5.10 NEW PASSWORD : XXXX setup:  (Default password : 0000)

This device allows user to set the new password.

3.5.10.1 Press ” Up ” or “ Down ” to choose NEW PASSWORD : XXXX setup.

3.5.10.2 Press ” ENTER ” to confirm NEW PASSWORD : XXXX setup.

3.5.10.3 Press  ” Up ” or “ Down ” to choose number, and press“ Right ” or “ Left ” to move digit location.

3.5.10.4 Press ” MENU ” to exit and confirm current operation.

3.5.10.5 Press ” MENU ” again to exit and close SYSTEM setup mode.

3.5.11 CLEAR HDD setup

This device allows user to have the HDD cleared.

3.5.11.1 Press ” Up ” or “ Down ” to choose CLEAR HDD setup.

3.5.11.2 Press ” ENTER ” to confirm CLEAR HDD setup.

3.5.11.3 Press ” Up ” or “ Down ” to choose CLEAR HDD setup MASTER or SLAVE.

The screen will show followings.

|  | ALL DATA IN HDD WILL BE CLEARED ARE YOU SURE? (◄ : NO ► : YES ) |
| --- | --- |

Press“ ►” to clear HDD

NO: Confirm not to clear HDD.

3.5.11.4 Press ” MENU ” to exit and confirm current operation.

3.5.11.5 Press ” MENU ” again to exit and close SYSTEM setup mode.

3.5.12 SYSTEM RESET setup

This device allows user to reset the system.

3.5.12.1 Press ” Up ” or “ Down ” to choose SYSTEM RESET setup.

3.5.12.2 Press ” ENTER” to confirm SYSTEM RESET setup.

3.5.12.3 Press ” ” Up ” or “ Down ” to choose SYSTEM RESET Yes or No

YES: To confirm System Reset (load default system reset)

NO: Confirm not to System Reset

3.5.12.4 Press ” MENU ” to exit and confirm current operation.

3.5.12.5 Press ” MENU ” again to exit and close SYSTEM setup mode.

3.5.13 REMOTE MODE setup

This device allows user to set the remote mode.

3.5.13.1 Press ” Up ” or “ Down ” to choose REMOTE MODE setup

3.5.13.2 Press ” ENTER ” to confirm REMOTE MODE setup

3.5.13.3 Press  ” Up ” or “ Down ” to choose Interface for REMOTE MODE setup

RS-232:  Use RS-232 interface

RS-485:  Use RS-485 interface

3.5.13.4 Press ” MENU ” to exit and confirm current operation.

3.5.13.5 Press ” MENU ” again to exit and close SYSTEM setup mode.

3.5.14 Remote protocol Transmitting Baud Rate setup

This device allows user to set the remote protocol transmitting baud rate.

3.5.14.1 Press ” Up ” or “ Down ” to choose Baud Rate setup.

3.5.14.2 Press ” ENTER ” to confirm

3.5.13.3 Press  ” Up ” or “ Down ” to choose BAUD RATE setup

115200、57600、19200、9600、4800、3600、2400、1200

3.5.14.4 Press ” MENU ” to exit and confirm current operation.

3.5.14.5 Press ” MENU ” again to exit and close SYSTEM setup mode.

3.5.15 Remote protocol ID setup:

This device allows user to control different DMR by setting RS232 remote protocol.    ID number can be set from 000 to be 255.

3.5.15.1 Press ” Up ” or “ Down ” to choose ID setup.

3.5.14.2 Press ” ENTER ” to confirm ID setup

3.5.15.3 Press ” Up ” or “ Down ” to set numerical number, from 000 ~ 255

3.5.15.4 Press ” MENU ” to exit and confirm current operation.

3.5.15.5 Press ” MENU ” again to exit and close SYSTEM setup mode.

3.6   SEARCH setup

| Press ”MENU” to enter main menu. You will need to enter password to access main menu. Press “ Right ” or “ Left ” to move digit, and to press Up ” or “ Down ” to select number. Press ”ENTER” button to confirm password. Ex.: Password: 0000 (Default: 0000) | PASSWORD: 0000 |
| --- | --- |

| After keying in correct password, and confirm by pressing ”ENTER” button, screen will show following options. SEARCH ------- Find recorded list TIMER -------- Scheduling Record RECORD------- Record Mode Setup CAMERA ------- Camera Channel Setup SYSTEM ------- System Setup EVENT -------- Event List | (MENU) ► SEARCH TIMER RECORD CAMERA SYSTEM EVENT |
| --- | --- |

3.6.1 LAST RECORD for recorded video

| Press ” Up ” or “ Down ” to choose LAST RECORD item. Following screen will be displayed. Press “ENTER” to confirm LAST RECORD. | ►LAST RECORD FULL LIST ALARM LIST TIME SEARCH |
| --- | --- |

3.6.2 FULL LIST for recorded video

| Press ” Up ” or “ Down ” to choose Full List item. Following screen will be displayed. | LAST RECORD ►FULL LIST ALARM LIST TIME SEARCH |
| --- | --- |

| Press “ ENTER ” to confirm Full List, and the following screen will be displayed. Press ” Up ” or “ Down ” to choose wanted recorded event (There will be shown only 8 list in one page.), and press “Right ” or “ Left ” to change the pages. Press “ENTER” to Play selected recorded video | ►M 2002-JAN-01 01:02:03 M-HDD M 2002-JAN-01 01:02:03 M-HDD A 2002-JAN-01 01:02:03 M-HDD T 2002-JAN-01 01:02:03 M-HDD T 2002-JAN-01 01:02:03 S-HDD M 2002-JAN-01 01:02:03 S-HDD ◄: PAGE UP►: PAGE DOWN |
| --- | --- |

M : Manual Record time / A : Alarm Record time

T : Timer Record time

M-HDD :  Storage in Master HDD

S-HDD :  Storage in Slave HDD

3.6.3 ALARM LIST for recorded video

| Press ” Up ” or “ Down ” to choose ALARM LIST. Following screen will be displayed. | LAST RECORD FULL LIST ►ALARM LIST TIME SEARCH |
| --- | --- |

| Press “ENTER” to confirm ALARM LIST, and the following screen will be displayed. Press ” Up ” or “ Down ” to choose wanted recorded event (There will be shown only 8 list in one page.), and press “ Left ” or “ Right” to change the pages. Press “ENTER” to Play selected recorded video | ►A 2002-JAN-01 01:02:03 M-HDD A 2002-JAN-01 01:02:03 M-HDD A 2002-JAN-01 01:02:03 M-HDD A 2002-JAN-01 01:02:03 M-HDD A 2002-JAN-01 01:02:03 S-HDD A 2002-JAN-01 01:02:03 S-HDD ◄: PAGE UP►: PAGE DOWN |
| --- | --- |

3.6.3 TIME SEARCH for recorded video

| Press” Up ” or “ Down ” to choose Time Search item. Following screen will be displayed. | LAST RECORD FULL LIST ALARM LIST ►TIME SEARCH |
| --- | --- |

| Press “ENTER” to confirm Full List, and the screen will be as right displayed. Press ” Up ” or “ Down ” to choose the wanted date & hour for recorded event , and press “ Left ” or “ Right ” to change the location. Press “ENTER” to play select time recorded video. If there is no record in the database, the screen will display “ NOT FOUND ” . | PLAY TIME: 2002-JAN-01 18 |
| --- | --- |

3.7   TIMER setup  (Schedule Time-Lapse record mode setup )

3.7.1 Enter TIMER setup

| Press ”MENU” to enter main menu. You will need to enter password to access main menu. To press “ Left ” or “Right ” to move digit, and press ” Up “ or “ Down ” to select number. Press ”ENTER” button to confirm password. Ex.: PASSWORD : 0000 (Default : 0000) | PASSWORD : 0000 |
| --- | --- |

| After keying in correct password, and confirm by pressing ”ENTER” button, following screen will be displayed. SEARCH ------- find recorded list TIMER ------- scheduling Record RECORD------- Record Mode Setup CAMERA ------- Camera Channel Setup SYSTEM ------- System Setup EVENT -------- Event List | (Menu) SEARCH ► TIMER RECORD CAMERA SYSTEM EVENT |
| --- | --- |

| Select TIMER item, and press “ENTER” button to confirm TIMER Record setup mode. Following screen will be displayed. | (TIMER) DAY START END QUALITY IPS DAILY 00:00 00:00 BEST 15A DAILY 00:00 00:00 BEST 15A DAILY 00:00 00:00 BEST 15A DAILY 00:00 00:00 BEST 15A DAILY 00:00 00:00 BEST 15A DAILY 00:00 00:00 BEST 15A DAILY 00:00 00:00 BEST 15A DAILY 00:00 00:00 BEST 15A TIMER ENABLE : NO |
| --- | --- |

3.7.2 TIMER setup:

3.7.2.1 Press ” ENTER ” to confirm TIMER setup.

3.7.2.2 Press ” Up ” or “ Down ” to choose the Timer Record day

Daily : Everyday

MON : Monday

TUE : Tuesday

WED : Wednesday

THU : Thursday

FRI : Friday

SAT : Saturday

SUN : Sunday

MO-FR : Monday to Friday

SA-SU : Saturday & Sunday

JAN-01 : Special Date

OFF : Not activated

3.7.2.3 Press “ Left ” or “ Right ” move to START record time 00:00 (HH:MM)

Press ” Up ” or “ Down ” to change START record time numerical digit

3.7.2.4 Press “ Left ” or “ Right ” move to END record time 00:00 (HH:MM)

Press ” Up ” or “ Down ” to change END record Time numerical digit

3.7.2.5 Press  “ Left ” or “ Right ” move to QUALITY

Press” Up ” or “ Down ” to choose options of

BEST, HIGH, NORMAL, BASIC

3.7.2.6 Press  “ Left ” or “ Right ” move to Record IPS (Images Per Second)

Press” Up ” or “ Down ” to choose options of

NTSC－15A、15、8、4、2、1

PAL－12A、12、6、3、2、1

3.7.2.7 Press ”MENU” to confirm current operation, and enter to next Timer Record

setup.

3.7.2.8 Press “ UP ” or “ DOWN ” move location to Timer Enable：Yes or No setup

YES : To confirm Timer Enable：Scheduling Timer Record function

NO : To confirm no Timer Enable：No Scheduling Timer Record function

3.7.2.9 Press ” MENU ” to exit and confirm current operation.

3.7.2.10 Press again ” MENU ” to exit and close TIMER setup mode.

3.8  RECORD setup

3.8.1 To Enter “ Record “ setup

| Press ”MENU” to enter main menu. You will need to enter password to access main menu. To press “ Left ” or “ Right ” to move digit, and to press ” Up ” or “ Down ” to select number. To press ” ENTER ” button to confirm password. Ex.: PASSWORD: 0000 (Default: 0000) | PASSWORD: 0000 |
| --- | --- |

| After keying in correct password, and confirm by pressing ” ENTER ” button, following screen will be displayed.SEARCH ------- find recorded list TIMER ------- scheduling Record RECORD------- Record Mode Setup CAMERA ------- Camera Channel Setup SYSTEM ------- System Setup EVENT -------- Event List | (MENU) SEARCH TIMER ► RECORD CAMERA SYSTEM EVENT |
| --- | --- |

| To choose Record item, and press “ ENTER ” to enter Record mode. Following screen will be displayed. | (RECORD) ► HDD OVERWRITE: NO RECORD IPS: 15A RECORD QUALITY : NORMAL ALARM REC IPS: 15A ALARM REC QUALITY : NORMAL |
| --- | --- |

3.8.2 HDD OVERWRITE setup :

3.8.2.1 Press ” ENTER” to confirm HDD OVERWRITE setup.

3.8.2.2 Press ” Up ” or “ Down ” to choose HDD OVERWRITE.

NO : When HDD full will stop recording

YES : When HDD full will overwrite the HDD recording

3.8.2.3 Press ” MENU ” to exit and confirm current operation.

3.8.2.4 Press again ” MENU ” to exit and close HDD OVERWRITE setup mode.

3.8.3 RECORD IPS setup:

3.8.3.1 Press ” ENTER” to confirm RECORD IPS setup.

3.8.3.2 Press ” Up ” or “ Down ” to choose IPS record speed.

NTSC－15A、15、8、4、2、1

PAL－12A、12、6、3、2、1

3.8.3.3 Press ” MENU ” to exit and confirm current operation.

3.8.3.4 Press ” MENU ” again to exit and close RECORD IPS setup mode.

3.8.4 RECORD QUALITY setup

3.8.4.1 Press ” ENTER ” to confirm Record Quality setup.

3.8.4.2 Press” Up ” or “ Down ” to choose Record Quality level.

BEST, HIGH, NORMAL, BASIC

3.8.4.3 Press ” MENU ” to exit and confirm current operation.

3.8.4.4 Press ” MENU ” again to exit and close RECORD QUALITY setup mode.

3.8.5 ALARM REC IPS setup

3.8.5.1 Press ” ENTER ” to confirm ALARM REC IPS setup.

3.8.5.2 Press ” Up ” or “ Down ” to choose ALARM REC IPS record speed.

NTSC－15A、15、8、4、2、1

PAL－12A、12、6、3、2、1

3.8.5.3 Press ” MENU ” to exit and confirm current operation.

3.8.5.4 Press again ” MENU ” to exit and close ALARM REC IPS setup mode.

3.8.6 ALARM REC QUALITY setup

3.8.6.1 Press ” ENTER ” to confirm ALARM REC QUALITY setup.

3.8.6.2 Press” Up ” or “ Down ” to choose ALARM REC QUALITY level.

BEST, HIGH, NORMAL, BASIC

3.8.6.3 Press ” MENU ” to exit and confirm current operation.

3.8.6.4 Press again ” MENU ” to exit and close ALARM REC QUALITY setup mode.

3.9  CAMERA setup

| Press ”MENU” to enter main menu. You will need to enter password to access main menu.Press “ Left ” or “ Right ” to move digit, and to press ” Up ” or “ Down ” to select number. To press ” ENTER ” button to confirm password. Ex.: Password : 0000 (Default : 0000) | PASSWORD : 0000 |
| --- | --- |

| After keying in correct password, and confirm by pressing ” ENTER ” button, Following screen will be displayed.SEARCH ------- find recorded list TIMER ------- scheduling Record RECORD------- Record Mode Setup CAMERA ------- Camera Channel Setup SYSTEM ------- System Setup EVENT -------- Event List | (MENU) SEARCH TIMER RECORD ► CAMERA SYSTEM EVENT |
| --- | --- |

CAMERA setup

|  | TITLE |  | DWELL |  |  |  | ALARM | RECORD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ► | ---- | 01 | ON | 5 | 5 | 5 | LOW | EVENT |
|  | ---- | 02 | ON | 5 | 5 | 5 | LOW | EVENT |
|  | ---- | 03 | ON | 5 | 5 | 5 | LOW | EVENT |
|  | ---- | 04 | ON | 5 | 5 | 5 | LOW | EVENT |
|  | ---- | 05 | ON | 5 | 5 | 5 | LOW | EVENT |
|  | ---- | 06 | ON | 5 | 5 | 5 | LOW | EVENT |
|  | ---- | 07 | ON | 5 | 5 | 5 | LOW | EVENT |
|  | ---- | 08 | ON | 5 | 5 | 5 | LOW | EVENT |
|  | ---- | 09 | ON | 5 | 5 | 5 | LOW | EVENT |
|  | ---- | 10 | ON | 5 | 5 | 5 | LOW | EVENT |
|  | ---- | 11 | ON | 5 | 5 | 5 | LOW | EVENT |
|  | ---- | 12 | ON | 5 | 5 | 5 | LOW | EVENT |
|  | ---- | 13 | ON | 5 | 5 | 5 | LOW | EVENT |
|  | ---- | 14 | ON | 5 | 5 | 5 | LOW | EVENT |
|  | ---- | 15 | ON | 5 | 5 | 5 | LOW | EVENT |
|  | ---- | 16 | ON | 5 | 5 | 5 | LOW | EVENT |

3.9.1 TITLE :It allows you to assign a title to each camera input. Initially each title is the camera’s number.

3.9.1.1 Press” Up ” or “ Down ” : Select channel in the Channel setup

3.9.1.2 Press ENTER: Confirm the channel

3.9.1.3 Press “ Left ” or  “ Right ” : Select the location of title (6 characters maximum)

3.9.1.4 Press ” Up ” or “ Down ” : Select the characters for display (0~9, A~Z,-, :, (, ), )

3.9.1.5 Press “ MENU ” :  Confirm and exit the title setting

3.9.1.6 Press “ MENU ” :  Exit the Channel setup

3.9.2 DWELL: to set up auto switching channel

3.9.2.1 Press ” Up ” or “ Down ” : Select channel in the Channel setup

3.9.2.2 Press “ ENTER ”:  Enter the channel setup

3.9.2.3 Press “ Left ” or  “ Right ” : Select DWELL on the screen

3.9.2.4 Press  ” Up ” or “ Down ” : Select ON/OFF on the screen

3.9.2.5 Press “ MENU ” :  Confirm and exit the dwell setting

3.9.2.6 Press “ MENU ” :  Exit the channel setup

3.9.3   (Brightness) /  (Contrast) /  (Color): video adjustment of each channel

3.9.3.1 Press ” Up ” or “ Down ” : Select channel in the Channel setup

3.9.3.2 Press “ENTER” Enter the channel setup

3.9.3.3 Press “ Left ” or  “ Right ” : Select brightness/contrast/color on the screen

3.9.3.4 Press ” Up ” or “ Down ” : Set up 0~9 on the screen.

3.9.3.5 Press  “MENU ” :  Confirm and exit the brightness/contrast/color setting.

3.9.3.6 Press  “MENU”:  Exit the channel setup

3.9.4 ALARM

3.9.4.1 Press ” Up ” or “ Down ” : Select channel in the Channel setup

3.9.4.2 Press “ENTER”  Enter the cannel setup

3.9.4.3 Press “ Left ” or  “ Right ” : Select ALARM on the screen

3.9.4.4 Press ” Up ” or “ Down ” : Select LOW/OFF/HIGH for alarm polarity

3.9.4.5 Press  “MENU”:” Confirm and exit the alarm setting.

3.9.4.6 Press  “MENU”:” Exit the channel setup

3.9.5 RECORD

3.9.5.1 Press ” Up ” or “ Down ” : Select channel in the Channel setup

3.9.5.2 Press “ENTER” :  Enter the channel setup

3.9.5.3 Press ” “ Left ” or “ Right ” : Select Record on the screen.

3.9.5.4 Press ” Up ” or “ Down ” : Select VCR record method as below (VCR will only record the channel which has been set up for EVENT/ NORMAL/ OFF)

EVENT :  when alarm input is triggered, VCR will record alarming channel more frequently. For example : VCR normal record method is 1-2-3-4-5-……-16. In Event mode, when CH01 is triggered, the record method will become  1-2-1-3-1-4-1-5-…-16.

NORMAL :  when external alarm is on, VCR will record normally as set up.

OFF : DMR will not record

3.9.5 Press “ MENU ” :  Confirm and exit the record setting

3.9.6 Press “ MENU ”:  Exit the channel setup

3.10   MOTION DETECTION setup

3.10.1 Press “ MENU “ to enter the menu set up, then “ Down” to CAMERA setup,         Press “ENTER” twice to enter the  Motion Detection Setup .

3.10.2 Each screen displays the current camera picture overlaid with the motion targets (as Figure 1). You can push the button “ Left ” or “ Right ”, ” Up ” or “ Down ” to adjust   motion detection in ON or OFF.

3.10.3 The targets on each motion setup can be turned to ON or OFF individually by row. To set up targets, using the following front panel buttons:

3.10.3.1 Press ‘ENTER” to confirm the channel

3.10.3.2 Press “ENTER” to enter motion mode

3.10.3.3 ▲ Up -- moves the target cursor up one row at a time.

3.10.3.4 ▼ Down -- moves the target cursor down one row at a time.

3.10.3.5 ◄ Left -- moves the target cursor left one column at a time.

3.10.3.6 ► Right -- moves the target cursor right one column at a time.

3.10.3.7 Press “ENTER” to turn the target cursor ON

Press “ENTER” again to turn the target cursor OFF

3.10.3.8 Camera Select (1-15) -– toggles the corresponding target on the cursor line ON or OFF.  (Figure 1) There are only 15 targets in a row, therefore; only Camera Selected (1-15) can be operated.

3.10.3.9 Zoom – turns all targets in the current row ON or OFF. (Figure 1-2)

3.10.3.10 PIP – turns all targets on the screen ON or OFF. (Figure 1-3)

3.10.4 Press “ SLOW “ button to setup the Sensitivity list up to 255

Press “ REC “ button to setup the Sensitivity list down to 000

Sensitivity value is related to motion and brightness change.

Low value (as 001) means higher sensitivity on motion and brightness change.

High value (as 255) means lower sensitivity on motion and brightness change.

User can choose the suitable sensitivity value in different locations.                    The default value is set on 32.

Note:

If Motion Detection is set, and to be activated, there will be 2 actions as followings:

1). Live video - the motion detected camera(s) will be scanned more frequently, if the Video set up for 16 cameras, the sequence to scan the cameras are from 1st, 2nd, 3rd... until 16th cameras, but if the motion is detected on Camera #1, the sequence to scan #1 camera will be more frequently. It will be scanning as 1st, 2nd, 1st, 3rd, 1st, 4th...1st, 16th. And CH 1 will show

The motion-detected camera will be scanned more frequently, which will be one after another camera been detected. If 2nd camera and 3rd camera both motion detection are activated, they will be scanning as 1st, 2nd, 3rd, 2nd, 2nd, 3rd, 3rd, 2nd, 3rd, 4th, 2nd, 3rd....3rd, 16th, 2nd, 3rd and vice versa. And CH2, CH3 will show  until motion function stop.

2). Recorded Video - more frequently video will be recorded for the motion detected cameras, function same as Live video in the above mentioned.

Basically, the motion detection will only change the scanning sequence while in live mode or in Record mode. It only show  on monitor and it cannot automatically from stand by mode to activate Record Mode.

Figure 1   MOTION DETECTION SETUP

| 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  | 11 |  | 12 |  | 13 |  | 14 |  | 15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 032 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |

Figure 1-1   MOTION DETECTION SETUP – 1~16

| 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  | 11 |  | 12 |  | 13 |  | 14 |  | 15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 032 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |

Figure 1-2   MOTION DETECTION SETUP – LINE

| 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  | 11 |  | 12 |  | 13 |  | 14 |  | 15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 032 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |

Figure 1-3   MOTION DETECTION SETUP – ALL

| 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  | 11 |  | 12 |  | 13 |  | 14 |  | 15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 032 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |  | -- |

3.11 Event

3.11.1 Enter “ Event “ list

| Press ”MENU” to enter main menu. You will need to enter password to access main menu. Press “ Left ” or “ Right ” to move digit, and to press” Up ” or “ Down ” to select number. To press ”ENTER” button to confirm password. Ex.: Password: 0000 (Default: 0000) | PASSWORD : 0000 |
| --- | --- |

| After keying in correct password, and confirm by pressing ”ENTER” button, Following screen will be displayed. SEARCH ------- find recorded list TIMER ------- scheduling Record RECORD------- Record Mode Setup CAMERA ------- Camera Channel Setup SYSTEM ------- System Setup EVENT -------- Event List | (MENU) SEARCH TIMER RECORD CAMERA SYSTEM ► EVENT |
| --- | --- |

| Press “ENTER” to confirm Full List, and the screen will be as right displayed. Press ” ” Up ” or “ Down ” to choose wanted recorded event (Only 8 recorded events will be shown on a single page), and press “ Up ” or “ Down ” to change the pages. Press “ENTER” to Play selected recorded video | M–HDD WARNING 2002-JAN-01 03:00:00 M–HDD LOSS 2002-JAN-01 03:00:00 M–HDD ERROR 2002-JAN-01 03:00:00 S–HDD WARNING 2002-JAN-01 03:00:00 HDD FULL 2002-JAN-01 03:00:00 SYSTEM ERROR 2002-JAN-01 03:00:00 ----02 VLOSS 2002-JAN-01 03:00:00 ----03 ALARM 2002-JAN-01 03:00:00 ◄: PAGE UP ►: PAGE DOWN◄+►: CLEAR |
| --- | --- |

M-HDD WARNING: Master HDD might be failed.

M-HDD LOSS: Master HDD does not exist.  Now user can use the other HDD.

M-HDD ERROR: Master HDD might be error

S-HDD WARNING: Slave HDD might be failed

S-HDD LOSS: Slave HDD does not exist.  Now user can use the other HDD.

S-HDD ERROR: Slave HDD might be error

HDD FULL: HDD is full

SYSTEM ERROR: System might be failed

----02 VLOSS: Channel: 2 Video loss

----03 ALARM: Channel: 3 External I/O Alarm have triggered

POWER RESTORE : Power restore

4. Operation

  - Power on

Before powering on the unit, make sure the HDD has been locked, and the POWER LED is red color now. After pressing POWER button, POWER LED will turn to orange color, then all other LED light will turn to RED color except the LED for HDD. OSD screen will display “HDD Detecting”, Power on running time will take approximately 15 to 20 seconds. If HDD is set as Master, and screen will display “ MASTER HDD CONNECTED”. If HDD is set as Slave, and screen will display “SLAVE HDD CONNECTED”.  In order to shorten the power on running time, we suggest set the HDD as Master. After set DMR power on, POWER LED will turn to green color. (You can set up HDD to be Master or Slave by jumper setting on HDD.)

  - Recording

Your DMR offers a variety of flexible recording modes. You can set it up to record continually, scheduled time, or by record events, and you can also set up recording speed and resolution. You can set these options by selecting MENU and System before recording. Under the recording status, if power is stopped accidentally, recorded video will still store in the HDD after power on. DMR will continue with original recording setup after power return.

There are 3 record modes for DMR: Alarm Record, Timer Record, and Manual Record.  All record Modes descriptions in followings.

4.2.1 Alarm Record

When DMR is triggered by alarm input, it will start to record immediately.       Following screen will be displayed.

| A : Alarm trigger OW : HDD Over Write 032GB : If the OW location shows 32GB, it means that 32 GB HDD left for video recording. (1 HDD installed) 001GB : If the OW location shows 32GB, it means 032GB that 32 GB left in slave HDD and 1 GB left in master HDD for video recording. (2 HDD installed) | 2002 – JAN –01 01:02:03 A●OW |
| --- | --- |

4.2.2 Timer Record

DMR will follow Timer setup to record, and the recording speed and quality will be followed TIMER recording mode setup in main menu.

| The screen will be as right displayed. T : Timer record OW : HDD Over Write 032GB : If the OW location shows 32GB, it means that 32 GB HDD left for video recording. (1 HDD installed) 001GB : If the OW location shows 32GB, it means 032GB that 32 GB left in slave HDD and 1 GB left in master HDD for video recording. (2 HDD installed) | 2002 – JAN –01 01:02:03 T●OW |
| --- | --- |

4.2.3 Manual Record

You can press “REC” to start recording immediately. The recording speed & quality will be followed by “Record” recording mode setup in main menu.

| Following screen will be displayed. M : Manual record OW : HDD Over Write 032GB : If the OW location shows 32GB, it means that 32 GB HDD left for video recording. 001GB : If the OW location shows 32GB, it 032GB means that 32 GB left in slave HDD and 1 GB left in master HDD for video recording. (2 HDD installed) | 2002 – JAN –01 01:02:03 032GB M●001GB |
| --- | --- |

  - Camera Select (1-16)

Press Camera Select (1-16) to select appointed camera to display on full screen mode.

  - Select

This device allows user to get the ideal view size.

    - Press     ,,      ,      button to make different view size.

    - Press “ Select ” then “ Left ” or “ Right ” button to locate the desired position.

    - Press “ Camera Select (1-16) “ to select the appointed camera.

    - Press “ Menu “ to exit.

4.5  Piture in Picture (PIP)

Press PIP       button to enter PIP display screen. The PIP format displays a full screen “background” picture with a 1/16 size screen ‘insert”.

4.5.1 Press       button to display the last selected PIP mode.

4.5.2 Press “ Select ” then “ Left ” or “ Right ” button to move the insert screen.

4.5.3 Press “ Menu “ to exit

4.6  Zoom

Press ZOOM button to enlarge the display of main picture. It displays zoom picture on main picture and a small window inserted. The inserted window contains a movable 1/4 view size of the appointed camera.

4.6.1 Press the appointed “ Camera Select (1-16) “.

4.6.2 Press “ Zoom “ button to be zoom mode.

4.6.3 Press the “Zoom ” button again to move the zoom pointer.

4.6.4 Press “ Menu “ to exit.

4.7  Play

Press “ PLAY ” then DMR will enter the Play mode; it will show the last record.

4.7.1 Fast Forward (F.F. ) & Fast Rewind (F.R.)

4.7.1.1 Press “ PLAY ” and press ” ►►” for fast forward searching screen. Press once ” ►►” to get speed for 2X. To press twice “►►” to get speed for 4X,. …And the maximum speed can be 32X.

4.7.1.2 Press “ PLAY ” then Press ”◄◄” for fast rewind searching screen. Press

once “◄◄ ” to get speed for 2X. To press twice “◄◄ ” to get speed for 4X, … and the maximum speed can be 32X.

4.7.2 Slow Forward (S.F.) & Slow Rewind (S.R.)

4.7.2.1 Press “ PLAY ” then press ” SLOW ” for slow forward, To press once “►►” for slow forward speed 1/2X. and press twice “►►” speed for 1/4X, ….the minimum slow speed can be 1/32X.

4.7.2.2 Press “ PLAY ” then press “ SLOW ” for slow rewind, To press once ” ◄◄ ” for slow rewind speed 1/2X, and press twice “◄◄” speed for 1/4X, ….the minimum slow speed can be 1/32X.

4.7.3 Pause : Press “ PLAY ” then press “ PAUSE ”, it will pause the screen

4.7.4 Stop : Press “ STOP ” under any circumstance, it will return DMR to live monitoring mode.

4.7.5 Image jog dial

4.7.5.1 Press “ PLAY ” then press “ PAUSE ”, it will pause the screen. Then press    “►► ” for single image play, it will show recorded image one by one when continuously pressing the ” ►► ”.

4.7.5.2 Press “ PLAY ” then press “ PAUSE ”, it will pause the screen. Then press “◄◄ ”for single image play, it will show recorded image one by one by continuously pressing the ” ◄◄ ”, (You may not see any image updated on screen after continuously pressing the ” ◄◄ ” in the beginning. You will need to continue pressing ” ◄◄ ” until all current images on screen have been pass through. The updated images will only be seen after pass through 1 cycle from paused images. )

  - VIDEO LOSS

Screen will display “    ” if the video input is not connected properly.

  - KEY LOCK WITH PASSWORD

Press “ MENU ” and “ ENTER “ key at same time, Screen will display “ KEY LOCK ” it will be    going to the Key Lock function.

Press “MENU” and “ENTER “ key again at the same time, key in password (Default: 0000) and press “ ENTER”, Screen will display “ KEY UNLOCK ” it will stop the Key Lock function.

5. Trouble shooting & Appendix

  - Trouble shooting

When DMR appears a malfunction; it may be not serious and can be corrected. The table below describes some typical problems and their remedies.  Check them before calling your DMR dealer:

| PROBLEM | SOLUTION |
| --- | --- |
| HDD Not Found | - Please Insert HDD - Please use the Key to lock the HDD Cartridge And press any key |
| No power | - Check power cord connections. - Confirm that there is power at the outlet. |
| Not working when press any button | - Check if it is under Key Lock mode. - Press “MENU” & “ENTER” at same time to exist Key Lock mode. |
| No recorded video | Check if the HDD is installed properly. |
| Timer Record enable does not working | Check if the Record Enable is set to YES |
| No live video | - Check camera video cable and connections. - Check monitor video cable and connections. - Confirm that the camera has power. - Check camera lens setting. |

  - Compatible HDD brands

| Manufacturer | Model | Capacity | Rotation |
| --- | --- | --- | --- |
| HITACHI | Deskstar 180 GXP (120 GB) | 120GB | 7200rpm |
| IBM | Deskstar 120GXP (40GB) | 40GB | 7200 rpm |
| IBM | Deskstar 60GXP IC35l060 | 60GB | 7200 rpm |
| IBM | Deskstar 120GXP (80GB) | 80GB | 7200 rpm |
| IBM | Deskstar 120GXP (120GB) | 120GB | 7200 rpm |
| Maxtor | DiamondMax 536DX(60GB) 4W060H4 | 60GB | 7200rpm |
| Maxtor | DiamondMax Plus 9, Model#6Y120L | 120GB | 7200 rpm |
| Seagate | Barracuda ATA IV, ST340016A | 40GB | 7200 rpm |
| Seagate | Barracuda ATA V, ST3120023A | 120GB | 7200 rpm |
| Seagate | Barracuda ATA IV, ST380021A | 80GB | 7200rpm |
| Western Digital | Caviar WD400BB-00BSA0 | 40GB | 7200 rpm |
| Western Digital | Caviar WD400EB-00CPF0 | 40GB | 5400 rpm |
| Western Digital | Caviar WD1200BB-00CAA1 | 120GB | 7200rpm |